| HYDRAULIC ANALYSIS AND STRUCTURE SELECTION | Project No. PCN XX-0-000(000)000 12345 Existing Structure ####-###.### (Enter Existing Structure ID) Proposed Structure ####-###.### (Enter Proposed Structure ID) XX Prepared by NORTH DAKOTA DEPARTMENT OF TRANSPORTATION BISMARCK, NORTH DAKOTA http://www.dot.nd.gov/ DIRECTOR Ronald J. Henke, P.E. PROJECT DEVELOPMENT DIRECTOR Jon Ketterling, P.E. Principal Author: (Consulting Firm OR name, NDDOT Division name) MONTH, YEAR |
| --- | --- |

23 USC § 407

NDDOT Reserves All Objections

XX-0-000(000)000

PCN 12345

NAME OF FEATURE INTERSECTED (e.g. SPRING CREEK)

PROJECT LOCATION (e.g. NORTH OF JCT US 52)

COUNTY

CERTIFICATION

Contents

1	Introduction	1

2	Existing Structure	2

2.1	Location	2

2.2	Project Construction History	2

2.3	Existing Conditions	3

3	Hydrology & Hydraulics	4

3.1	Flood Frequency Discharges	4

3.2	Minimum Flood Frequency	5

3.3	Site Conditions	5

3.4	Floodplain/Floodway/Permitting	5

4	Structure Selection	7

4.1	Project Strategy/Scope of Work	7

4.2	Hydraulic Modeling	7

4.3	Alternatives Considered	8

4.4	Summary of Recommendations	9

4.5	Channel Excavation/Construction Impacts (if applicable)	11

4.6	Scour Analysis	12

4.7	Miscellaneous	13

Appendices

Appendix A: Hydraulic Calculation Data

Appendix B: Estimated Flood Extents

Appendix C: Structure Comparison Summary

Appendix D: Hydraulic Design Data for Bridges or Hydraulic Design Data for Culverts

Appendix E: Watershed Area Map

Appendix F: Structure Inspection Report

Appendix G: FEMA FIRM Map

Appendix H: Miscellaneous

# Introduction

This report documents the hydraulic analysis of the proposed replacement of Structure ####-###.###. Information about the (study area, methodology, existing/proposed condition modeling results, etc) will be discussed.

Describe purpose of report.

# Existing Structure

## Location

The existing structure is a (structure description) with a skew of __ degrees. The structure is located (enter generalized location, e.g. 4 Miles South of Jct US 12) at the (cardinal direction)¼ Section __, Township __N, Range __W in __ County. The structure conveys the flows of (enter features intersected) under Highway __ at reference point __. The section of highway being evaluated is classified as (enter functional classification).

Describe location and include project location map identifying stream crossings, structure location, intersecting roadways, nearby towns, other relevant data.

85

Structure 0085-012.746

3-span 80’x28’ Concrete Slab Bridge

Spring Creek

Figure _: Project Location Map

## Project Construction History

The existing (enter structure description) was constructed in ____. The structure was (rehabilitated, widened, extended, etc) in ____ (delete as applicable).

The structure ID of the existing structure is ####-###.###. The structure ID of the proposed structure is ####-###.###. The existing structure ID will be used for the remainder of the report.

Note the existing structure’s construction year and briefly discuss construction and maintenance history. Specify the existing structure ID number and the new structure ID number. The new structure ID will be assigned when the project is programmed – contact NDDOT Bridge Division to obtain.

## Existing Conditions

The existing structure has an overall length of __ feet and a clear roadway width of __ feet. The existing structure has a measured minimum vertical clearance of __ feet (delete as applicable). The structure was last inspected in ____. At the time, the (delete following items as applicable) culvert was assigned a condition rating of _ (enter code description, i.e. good/fair/poor/etc condition), the bridge deck was assigned a condition rating of _ (___ condition), the superstructure was assigned a condition rating of _ (___ condition), and the substructure was assigned a condition rating of _ (___ condition). The channel was assigned a condition rating of _ (___ condition). Defects noted on the structure at the time included ___. A chaining survey was completed ____ and recorded to have ___% delamination (delete as applicable).

Skew: _ degrees

Current ADT (year) = (enter current ADT)

Future ADT (year) = (enter forecast ADT)

Fill out the appropriate table below:

Table _ – Structure Inventory (for existing bridges)

| Structure No. | Crossing | Size | Year Built | Condition Ratings |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | Deck | Super | Sub | Channel |
| ####-###.### | Feature Intersected (e.g. Spring Creek) | L’ x W’ (Insert Bridge Description) | #### | # | # | # | # |

OR:

Table _ – Structure Inventory (for existing culverts)

| Structure No. | Crossing | Size | Year Built | Condition Ratings |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  |  | Culvert | Channel |
| ####-###.### | Feature Intersected (e.g. Spring Creek) | Span’ x Rise’ x L’ or Diameter’ x L’ (Insert Culvert Description) | #### | # | # |

Specify existing structure details including type, size, location, skew, year built, waterway opening, condition ratings, AADT, etc. Make note of any observed scour or any other critical deficiencies.

# Hydrology & Hydraulics

## Flood Frequency Discharges

(ArcGIS, QGIS, etc) software was utilized to map the contributing drainage area using (topographic maps, LiDAR elevation data, etc). The drainage area for this structure was determined to be approximately ___ square miles with a stream gradient of ___ feet per mile and is located in Hydrologic Zone _. A map of the drainage area is provided in the appendix. Peak flood frequency discharge values were determined in accordance with USGS Scientific Investigations Report 2015-5096. The calculated results at the structure using the 2015 Regression Equations are shown below in Figure _. (Note that for ungaged sites on a gaged stream that has 10 or more years of peak-flow record, an area-weighted equation may be used as an alternative to the Regression Equations for an improved estimate)

Include brief description of drainage area size, stream gradient, Hydrologic Zone, roadway classification, and corresponding design flood frequency event. Include table of flood frequency discharge values (example utilizing 2015 Regression Equations provided below) and map of drainage area in appendix.

Figure _: USGS Regression Peak Discharges

## Minimum Flood Frequency

As per Article 89-14 of the North Dakota Administration Code (ND Stream Crossing Standards), based on the functional classification of (insert classification) for Highway ##, a ##-year minimum design flood frequency was used for this analysis. The minimum design flood frequency requirements as stated by ND Administrative Code 89-14 are shown below in Figure _.

Figure _: Minimum Design Flood Frequency

## Site Conditions

- The (drain, creek, river)’s watershed is situated in a (rural, urban, etc) area comprised of (insert description of contributing watershed, e.g. primarily cultivated land with some pastureland and numerous trees interspersed throughout). The stream channel for the (drain, creek, river) has a (insert description of stream slope, e.g. relatively mild, steep, etc) stream gradient.

- Describe the project site conditions and any special circumstances (upstream/downstream drainage structures, dams, levees, erosion, scour, sedimentation, channel changes, protected lands, residential structures, etc). Provide additional exhibits of unique features as figures or in appendix if necessary.

## Floodplain/Floodway/Permitting

- According to the FEMA Flood Map Service Center, Structure XXXX-XXX.XXX is situated within FEMA Flood Hazard Zone _, which is classified as the (enter FEMA zone description, i.e. Area of Minimal Flood Hazard, 100-yr Floodplain, etc). Therefore, a (Floodplain Authorization, Floodway Permit, etc) will/will not be required due to the proposed structure work. A USACE 404 Permit may be required if the proposed work results in impacts to jurisdictional wetlands or other waters. Include the following only if waterway is classified as Sovereign Lands: The (insert waterway name) is included in the State Navigable Waters list, and any proposed activities below the Ordinary High Water Elevation will require a Sovereign Lands Permit.

- Note if the project is located within or discharges into a floodplain/floodway. Discuss any permit requirements and include an exhibit of the FIRM map in the appendix.

# Structure Selection

## Project Strategy/Scope of Work

Project XX-0-000(000)000 is currently scheduled for a MM/DD/YY project completion date and has a listed bid opening date of MM/DD/YY. The project is expected to be constructed during the (enter year(s) of anticipated construction) construction season(s). The scope of work for the project includes (structure replacement/structure rehabilitation, reconstruction of a portion of the highway, etc).

Discuss project’s scope of work and strategy. Note any special circumstances related to the project that may affect the structure selection (e.g. addition of pedestrian lane on a bridge, incorporating wildlife crossing under a bridge, etc).

## Hydraulic Modeling

A 1D hydraulic model was created using (list software used and version number, e.g. FHWA’s HY-8 version 8.0.0 or USACE’s HEC-RAS version 6.6, and provide a brief summary of the software utilized). The software was used to prepare a 1D hydraulic model to simulate flow conditions for the existing structure(s) and _ proposed structure alternatives. The 1D hydraulic model included computations of (water surface profiles, velocities, flow rates, etc) at the structure and along (insert channel name) (delete as applicable).

For simplified hydraulic modeling such as HY-8:

The existing structure and the proposed structure alternatives were evaluated using HY-8. Peak discharges were input into the model to evaluate the existing structure and proposed structure alternatives at the (list flood events evaluated, e.g., 10-year, 25-year, 50-year, 100-year, 200-year, and 500-year flood events). A summary of the HY-8 analysis is included in the appendices, detailing the results for both the existing and proposed structures. This summary includes Crossing Summary Tables, Crossing Summary Front View Plots, Culvert Summary Tables, Water Surface Profile Plots, Culvert Data, Tailwater Data, and Roadway Data (delete or add items as applicable).

For more complex hydraulic modeling such as HEC-RAS (required for structure replacements of existing bridges or structure replacements within regulatory floodways):

The boundary limits of the model are shown in Figure _ below and extend approximately ____ feet upstream and ____ feet downstream of the existing structure crossing. (List source of land elevations used in model, e.g. topographic survey conducted by NDDOT, LiDAR observations obtained from the NDDWR, etc) were used to determine the land elevations within the model limits. At the channel, (cross-sectional data, bathymetric survey collected by NDDOT, etc) were utilized to depict the underwater topography within the model.

Cross sections were input into the HEC-RAS model to represent the geometry for both the existing structure(s) and the proposed structure alternatives. Steady-state/unsteady-state flow data were incorporated into the model to simulate the (list flood events evaluated, e.g., 10-year, 25-year, 50-year, 100-year, 200-year, and 500-year flood events). HEC-RAS plan simulations were then performed to evaluate the (channel/creek/river) under existing conditions and proposed conditions with the replacement structures at the provided flood events. Results of these simulations are included in the appendices and include (add or remove the following items as applicable): water surface profile plots, cross sections with projected water surface elevations at the structure(s), output summary tables for the analyzed reach, detailed crossing summary tables for the structure(s), graphical representations of the flooding extents at each recurrence interval, and scour plots (for proposed bridge structures).

85

Spring Creek

Structure 0085-012.746

80’x28’ Cont. Conc. Slab Bridge

Figure _: HEC-RAS Model Domain Boundary

Provide brief description of hydraulic modeling software and version used. Describe unique modeling circumstances and provide exhibit of model domain limits in Figure above. Briefly describe topographic/bathymetric survey conducted upstream and downstream of the structure crossing for modeling purposes. Note any summary items provided in the appendices.

## Alternatives Considered

(Insert Number of Alternatives) proposed structure alternatives were modeled in consideration for replacement of the existing structure.

If applicable, list any proposed structure alternatives which were previously considered but were later discarded (e.g. bridge alternative would require grade raise and increase risk of flooding to an upstream building, inadequate cover for aluminum box culvert alternative, etc) and reasons for their exclusion from further hydraulic analysis here. If not applicable, this paragraph may be deleted.

List each modeled alternative and provide details on the alternative. Details may include: number of spans/barrels, length & width dimensions of structure, skew, inverts, Manning’s n values at the structure, ineffective flow areas, culvert sinking requirements, wildlife crossings, girder depths, horizontal/vertical clearance provided (for bridges), grade raise/profile adjustments, guardrail requirements, etc.

The proposed structure alternatives were modeled and analyzed in accordance with the ND Stream Crossing Standards. Summarize the simulation results, including discussion on calculated headwater elevations, freeboard provided at the design and 100-year discharges (for bridges), design velocities at the structure, and overtopping discharges. Discuss whether the proposed structure alternative creates any increased risk of flooding of any upstream buildings/structures. A summary of the simulation data for the (insert flood events evaluated in model, e.g. 10-year, 50-year, 100-year, and 500-year flood events), which includes the observed discharge through the structure, upstream water surface elevations, and velocities through the structure, is provided below in Table _.

Table _ – Alternative 1 Simulation Results (include additional tables for multiple alternatives)

| Event | Total Discharge (cfs) | Existing |  |  | Alternative 1 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Discharge thru Structure (cfs) | Upstream WSE (ft) | Avg Vel thru Structure (cfs) | Discharge thru Structure (cfs) | Upstream WSE (ft) | Avg Vel thru Structure (cfs) |
| 10-yr | - | - | - | - | - | - | - |
| 50-yr | - | - | - | - | - | - | - |
| 100-yr | - | - | - | - | - | - | - |
| 500-yr | - | - | - | - | - | - | - |

The estimated cost the proposed structure alternative(s) is shown below in Table _ – Cost Estimate. (Indicate whether the estimated costs includes any incidental items such as riprap, guardrail, embankment or if the proposed estimate is solely for the cost of the replacement structure)

Table _ – Cost Estimate

| Alternative Number and Size | Estimated Structure Cost |
| --- | --- |
| Alternative #1 – 2-span, _’x_’ Steel Stringer Bridge | $###,### |
| Alternative #2 – Quad _’ x _’ x _’ RCB | $###,### |
| Alternative #3 – Dbl _’ x _’ x _’ Aluminum Box Culvert | $###,### |

## Summary of Recommendations

Based on the information above, it is recommended that Alternative _, a (enter structure description), be selected as the replacement alternative. A layout of the proposed structure and a water surface profile plot of the existing structure and the proposed alternative are shown below in Figures _ and _.

Document the selected alternative with supporting justification. Discuss requirements for the selected alternative (inverts, wingwall type, countersinking/end-slope requirements, wildlife crossings, etc). For proposed bridge alternatives, note the design velocity through the bridge (typically <5 ft/s) and freeboard provided at the design and 100-yr discharge events. For proposed culvert alternatives, note the design outlet velocity (typically <10 ft/s) and whether the box culvert satisfies allowable headwater requirements noted in the ND Stream Crossing Standards. Provide a layout view of the proposed alternative and a WSE plot comparing the existing structure and the chosen alternative.

Quad 12’ x 12’ x 72’ RCB

Pipestem Creek

30

Figure _: Proposed Structure Layout (Box Culvert)

Figure _: Proposed Structure Layout (Bridge)

Figure _: Water Surface Elevation Comparison

## Channel Excavation/Construction Impacts (if applicable)

The proposed structure opening is expected to be broader than the current channel bottom. Therefore, it is anticipated that there will be a need for channel excavation to transition from the existing channel bottom to the opening of the proposed structure. A layout is provided below in Figure _.

Describe necessary construction requirements and unique grading situations to accommodate the recommended structure alternative. Provide an exhibit of proposed structure layout with channel impacts as necessary. Ensure construction requirements described in Hydraulic Report are documented in the final design plans with clear instruction using plan notes, sheets, cross sections, etc.

153rd Ave SE

Elm River

Figure _: Channel Excavation

## Scour Analysis

For Proposed Bridges:

Scour calculations were computed using (FHWA’s Hydraulic Toolbox software, HEC-18 equations, etc) for each of the bridge alternatives. Calculations for the calculated scour (and if applicable, documentation of existing observed scour/existing channel cross sections) are shown in the appendices. A summary of the scour calculations is shown below in Table _. The scour calculations were performed under the assumption that no scour countermeasures are in place.

Table _ - Estimated Scour, Alternative _

| Flood Frequency * | Discharge (cfs) | Contraction Scour (ft) | Pier Scour (ft) | Abutment Scour (ft) |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  |  | Left | Right |
| 50-yr | - | - | - | - | - |
| 100-yr | - | - | - | - | - |
| 200-yr | - | - | - | - | - |

*Refer to HEC-18 – Table 2.1 for guidance on recommended minimum scour design frequencies.

Based on the scour analysis for Alternative _, it is recommended that the proposed bridge have NBI Item 113 for Scour Critical Bridges coded as 8 (indicating the bridge is stable under assessed or calculated scour conditions) and SNBI Item B.AP.03 Scour Vulnerability coded as A (indicating the bridge is determined to be stable for scour). {The engineer will need to determine scour ratings for the bridge. After review, if the bridge does not have an NBI rating of 8 or SNBI rating of A, please replace with the appropriate rating and explanation. Any state structure will be required to be stable for scour.}

It is recommended that a _” layer of riprap on geosynthetic material be placed at the bridge to protect the channel and structure from possible scour. As per guidance from HEC-23, the riprap should cover the abutment endslopes and extend into the channel _’ from the toe of the endslope. This riprap should extend _’ upstream and downstream of the bridge along the channel, measured from the edge of the bridge. This riprap meets the minimum requirements of the pier scour envelope. A layout of the proposed riprap protection for Alternative _ is shown in Figure _.

Discuss any history of scour at the current structure, if applicable, and provide supporting documents in the appendices. Perform scour calculations for bridge alternates in accordance with Chapter V-02 of the NDDOT Design Manual and document results for contraction, pier, and abutment scour at design/critical flood events. Indicate software/methods used to calculate scour and include applicable scour input/output data, assumptions, and envelope plot exhibits in the appendices. Indicate the NBI Item 113 Rating for Scour Critical Bridges/SNBI Item B.AP.03 Scour Vulnerability Code. Describe applicable scour countermeasures as necessary.

For Proposed Culverts:

Scour calculations were not performed for the box culvert alternatives, as specified in Chapter V-02.07 of the NDDOT Design Manual. Energy dissipators for Alternative _ were designed using methodology provided in HEC-14. It is recommended that a _” layer of riprap be placed in the channel at the ends of the box culvert aprons for a distance of 10 feet upstream and _ feet downstream to protect the channel and structure from possible scour. Calculations for the proposed energy dissipator are shown in the appendices. A layout showing the proposed riprap/energy dissipator is shown in Figure _.

Discuss any history of scour at the existing structure, if applicable, and provide supporting documents in the appendices. Perform calculations for energy dissipator design at the proposed culvert outlet. If a riprap apron is to be used, it is generally recommended that riprap be installed for a distance of 10’ upstream of the culvert, while HEC-14 equations should be used to calculate the riprap length downstream of the structure.

## Miscellaneous

A Type, Size, and Location (TS&L) meeting (was held on _ OR will be held prior to plan completion) to discuss the following items:

- Structure Selection & Location

- Riprap Requirements

- Utilities

- Disposal of Existing Structures

- Construction Phasing

- ROW Impacts

- Temporary Bypass/Detours

Indicate items planned for discussion at the TS&L meeting, or if the meeting has already taken place, document the date of the meeting and include discussion on items discussed.

Appendix A: Hydraulic Calculation Data

For simplified hydraulic modeling such as HY-8:

Include HY-8 printouts for the existing structure and the proposed alternatives. These printouts should typically include Crossing Summary Tables, Crossing Summary Front View Plots, Culvert Summary Tables, Water Surface Profile Plots, Culvert Data, Tailwater Data, and Roadway Data (delete or add items as applicable)

For more complex hydraulic modeling such as HEC-RAS (required for structure replacements of existing bridges or structure replacements within regulatory floodways):

Include water surface profile plots shown along stream channel (within bathymetry survey extents) for all flood frequency events. Include plots for existing conditions and all alternatives considered. Include upstream/downstream cross section plots for existing structure and all alternatives considered. Display all flood frequency events in cross section plots. Include tabular data/software report printouts comparing critical hydraulic information (e.g. water surface elevations, flow velocities, etc) of the modeled alternatives along stream channel near area where structure is being replaced.

Spring Creek Model Domain

Spring Creek Water Surface Elevation Plot - Existing

Spring Creek Water Surface Elevation Plot – Alternative 1

Existing Vs Proposed Profile Comparison

|  |  |  | Existing |  | Alternative 1 |  | Comparison |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| River Sta | Profile | Q Total (cfs) | WSE (ft) | Vel (fps) | WSE (ft) | Vel (fps) | ΔWSE (ft) | ΔVel (ft/s) |
| 7135 | 10-yr | 1175.60 | 2882.71 | 2.01 | 2882.71 | 2.01 | 0 | 0 |
| 7135 | 50-yr | 2366.30 | 2884.78 | 2.08 | 2884.78 | 2.08 | 0 | 0 |
| 7135 | 100-yr | 2923.30 | 2885.44 | 2.12 | 2885.44 | 2.13 | 0 | 0.01 |
| 7135 | 500-yr | 4241.50 | 2886.71 | 2.30 | 2886.70 | 2.31 | -0.01 | 0.01 |
| 6516 | 10-yr | 1175.60 | 2882.18 | 2.42 | 2882.18 | 2.42 | 0 | 0 |
| 6516 | 50-yr | 2366.30 | 2884.32 | 2.42 | 2884.32 | 2.43 | 0 | 0.01 |
| 6516 | 100-yr | 2923.30 | 2885.00 | 2.50 | 2884.99 | 2.50 | -0.01 | 0 |
| 6516 | 500-yr | 4241.50 | 2886.31 | 2.72 | 2886.29 | 2.73 | -0.02 | 0.01 |
| 5531 | 10-yr | 1175.60 | 2880.87 | 3.83 | 2880.86 | 3.83 | -0.01 | 0 |
| 5531 | 50-yr | 2366.30 | 2882.73 | 4.44 | 2882.71 | 4.46 | -0.02 | 0.02 |
| 5531 | 100-yr | 2923.30 | 2883.35 | 4.32 | 2883.33 | 4.35 | -0.02 | 0.03 |
| 5531 | 500-yr | 4241.50 | 2884.61 | 3.94 | 2884.56 | 4.02 | -0.05 | 0.08 |
| 5156 | 10-yr | 1175.60 | 2880.66 | 1.89 | 2880.66 | 1.89 | 0 | 0 |
| 5156 | 50-yr | 2366.30 | 2882.49 | 1.85 | 2882.47 | 1.86 | -0.02 | 0.01 |
| 5156 | 100-yr | 2923.30 | 2883.09 | 1.82 | 2883.07 | 1.84 | -0.02 | 0.02 |
| 5156 | 500-yr | 4241.50 | 2884.33 | 1.84 | 2884.27 | 1.87 | -0.06 | 0.03 |
| 5046 | 10-yr | 1175.60 | 2880.56 | 2.28 | 2880.56 | 2.28 | 0 | 0 |
| 5046 | 50-yr | 2366.30 | 2882.39 | 2.46 | 2882.38 | 2.48 | -0.01 | 0.02 |
| 5046 | 100-yr | 2923.30 | 2883.01 | 2.45 | 2882.98 | 2.47 | -0.03 | 0.02 |
| 5046 | 500-yr | 4241.50 | 2884.27 | 2.38 | 2884.20 | 2.43 | -0.07 | 0.05 |
| 4857 | 10-yr | 1175.60 | 2880.24 | 2.81 | 2880.24 | 2.81 | 0 | 0 |
| 4857 | 50-yr | 2366.30 | 2882.22 | 2.69 | 2882.20 | 2.71 | -0.02 | 0.02 |
| 4857 | 100-yr | 2923.30 | 2882.87 | 2.64 | 2882.83 | 2.67 | -0.04 | 0.03 |
| 4857 | 500-yr | 4241.50 | 2884.17 | 2.49 | 2884.10 | 2.54 | -0.07 | 0.05 |
| 4435 | 10-yr | 1175.60 | 2879.59 | 2.43 | 2879.58 | 2.43 | -0.01 | 0 |
| 4435 | 50-yr | 2366.30 | 2881.70 | 2.77 | 2881.66 | 2.80 | -0.04 | 0.03 |
| 4435 | 100-yr | 2923.30 | 2882.41 | 2.74 | 2882.36 | 2.78 | -0.05 | 0.04 |
| 4435 | 500-yr | 4241.50 | 2883.83 | 2.79 | 2883.74 | 2.85 | -0.09 | 0.06 |
| 4147 | 10-yr | 1175.60 | 2879.50 | 1.56 | 2879.49 | 1.57 | -0.01 | 0.01 |
| 4147 | 50-yr | 2366.30 | 2881.58 | 2.01 | 2881.55 | 2.03 | -0.03 | 0.02 |
| 4147 | 100-yr | 2923.30 | 2882.28 | 2.18 | 2882.23 | 2.20 | -0.05 | 0.02 |
| 4147 | 500-yr | 4241.50 | 2883.69 | 2.48 | 2883.59 | 2.52 | -0.10 | 0.04 |
| 3989 | 10-yr | 1175.60 | 2879.43 | 1.85 | 2879.42 | 1.86 | -0.01 | 0.01 |
| 3989 | 50-yr | 2366.30 | 2881.48 | 2.58 | 2881.44 | 2.59 | -0.04 | 0.01 |
| 3989 | 100-yr | 2923.30 | 2882.15 | 2.84 | 2882.10 | 2.86 | -0.05 | 0.02 |
| 3989 | 500-yr | 4241.50 | 2883.53 | 3.34 | 2883.42 | 3.39 | -0.11 | 0.05 |

Existing Vs Proposed Profile Comparison (continued)

|  |  |  |  | Existing |  |  |  | Alternative 1 |  |  |  | Comparison |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| River Sta | Profile |  | Q Total (cfs) | WSE (ft) |  | Vel (fps) |  | WSE (ft) |  | Vel (fps) |  | ΔWSE (ft) |  | ΔVel (ft/s) |  |
| 3785 | 10-yr |  | 1175.60 | 2879.26 |  | 2.54 |  | 2879.25 |  | 2.54 |  | -0.01 |  | 0.00 |  |
| 3785 | 50-yr |  | 2366.30 | 2881.28 |  | 3.28 |  | 2881.24 |  | 3.31 |  | -0.04 |  | 0.03 |  |
| 3785 | 100-yr |  | 2923.30 | 2881.95 |  | 3.49 |  | 2881.89 |  | 3.53 |  | -0.06 |  | 0.04 |  |
| 3785 | 500-yr |  | 4241.50 | 2883.34 |  | 3.77 |  | 2883.22 |  | 3.86 |  | -0.12 |  | 0.09 |  |
| 3598 | 10-yr |  | 1175.60 | 2879.12 |  | 2.92 |  | 2879.10 |  | 2.93 |  | -0.02 |  | 0.01 |  |
| 3598 | 50-yr |  | 2366.30 | 2881.07 |  | 4.01 |  | 2881.02 |  | 4.05 |  | -0.05 |  | 0.04 |  |
| 3598 | 100-yr |  | 2923.30 | 2881.73 |  | 4.27 |  | 2881.66 |  | 4.34 |  | -0.07 |  | 0.07 |  |
| 3598 | 500-yr |  | 4241.50 | 2883.10 |  | 4.66 |  | 2882.96 |  | 4.79 |  | -0.14 |  | 0.13 |  |
| 3538 | 10-yr |  | 1175.60 | 2879.11 |  | 2.31 |  | 2879.10 |  | 2.32 |  | -0.01 |  | 0.01 |  |
| 3538 | 50-yr |  | 2366.30 | 2881.06 |  | 3.27 |  | 2881.02 |  | 3.29 |  | -0.04 |  | 0.02 |  |
| 3538 | 100-yr |  | 2923.30 | 2881.71 |  | 3.68 |  | 2881.64 |  | 3.72 |  | -0.07 |  | 0.04 |  |
| 3538 | 500-yr |  | 4241.50 | 2883.06 |  | 4.31 |  | 2882.91 |  | 4.44 |  | -0.15 |  | 0.13 |  |
| 3280 | 10-yr |  | 1175.60 | 2878.72 |  | 3.09 |  | 2878.71 |  | 3.10 |  | -0.01 |  | 0.01 |  |
| 3280 | 50-yr |  | 2366.30 | 2880.59 |  | 3.81 |  | 2880.53 |  | 3.87 |  | -0.06 |  | 0.06 |  |
| 3280 | 100-yr |  | 2923.30 | 2881.23 |  | 4.09 |  | 2881.14 |  | 4.18 |  | -0.09 |  | 0.09 |  |
| 3280 | 500-yr |  | 4241.50 | 2882.59 |  | 4.73 |  | 2882.37 |  | 5.07 |  | -0.22 |  | 0.34 |  |
| 3183 | 10-yr |  | 1175.60 | 2878.57 |  | 3.18 |  | 2878.56 |  | 3.19 |  | -0.01 |  | 0.01 |  |
| 3183 | 50-yr |  | 2366.30 | 2880.36 |  | 4.25 |  | 2880.29 |  | 4.33 |  | -0.07 |  | 0.08 |  |
| 3183 | 100-yr |  | 2923.30 | 2881.01 |  | 4.42 |  | 2880.89 |  | 4.56 |  | -0.12 |  | 0.14 |  |
| 3183 | 500-yr |  | 4241.50 | 2882.45 |  | 4.31 |  | 2882.21 |  | 4.71 |  | -0.24 |  | 0.40 |  |
| 3095 | 10-yr |  | 1175.60 | 2878.45 |  | 3.46 |  | 2878.43 |  | 3.47 |  | -0.02 |  | 0.01 |  |
| 3095 | 50-yr |  | 2366.30 | 2880.20 |  | 4.61 |  | 2880.11 |  | 4.74 |  | -0.09 |  | 0.13 |  |
| 3095 | 100-yr |  | 2923.30 | 2880.85 |  | 4.84 |  | 2880.72 |  | 4.96 |  | -0.13 |  | 0.12 |  |
| 3095 | 500-yr |  | 4241.50 | 2882.27 |  | 5.12 |  | 2882.04 |  | 5.19 |  | -0.23 |  | 0.07 |  |
| 3017 | 10-yr |  | 1175.60 | 2878.47 |  | 2.05 |  | 2878.46 |  | 2.01 |  | -0.01 |  | -0.04 |  |
| 3017 | 50-yr |  | 2366.30 | 2880.24 |  | 2.87 |  | 2880.17 |  | 2.73 |  | -0.07 |  | -0.14 |  |
| 3017 | 100-yr |  | 2923.30 | 2880.88 |  | 3.18 |  | 2880.77 |  | 3.00 |  | -0.11 |  | -0.18 |  |
| 3017 | 500-yr |  | 4241.50 | 2882.26 |  | 3.71 |  | 2882.05 |  | 3.51 |  | -0.21 |  | -0.20 |  |
| 2943 | Bridge |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 2816 | 10-yr |  | 1175.60 | 2878.46 |  | 0.84 |  | 2878.46 |  | 0.84 |  | 0 |  | 0 |  |
| 2816 | 50-yr |  | 2366.30 | 2880.17 |  | 1.33 |  | 2880.17 |  | 1.33 |  | 0 |  | 0 |  |
| 2816 | 100-yr |  | 2923.30 | 2880.76 |  | 1.55 |  | 2880.76 |  | 1.55 |  | 0 |  | 0 |  |
| 2816 | 500-yr |  | 4241.50 | 2882.04 |  | 1.99 |  | 2882.04 |  | 1.99 |  | 0 |  | 0 |  |

Existing Vs Proposed Profile Comparison (continued)

|  |  |  | Existing |  | Alternative 1 |  | Comparison |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| River Sta | Profile | Q Total (cfs) | WSE (ft) | Vel (fps) | WSE (ft) | Vel (fps) | ΔWSE (ft) | ΔVel (ft/s) |
| 2584 | 10-yr | 1175.60 | 2878.34 | 2.54 | 2878.34 | 2.54 | 0 | 0 |
| 2584 | 50-yr | 2366.30 | 2880.03 | 3.06 | 2880.03 | 3.06 | 0 | 0 |
| 2584 | 100-yr | 2923.30 | 2880.62 | 3.24 | 2880.62 | 3.24 | 0 | 0 |
| 2584 | 500-yr | 4241.50 | 2881.90 | 3.50 | 2881.90 | 3.50 | 0 | 0 |
| 2123 | 10-yr | 1175.60 | 2878.07 | 1.77 | 2878.07 | 1.77 | 0 | 0 |
| 2123 | 50-yr | 2366.30 | 2879.59 | 2.59 | 2879.59 | 2.59 | 0 | 0 |
| 2123 | 100-yr | 2923.30 | 2880.13 | 2.93 | 2880.13 | 2.93 | 0 | 0 |
| 2123 | 500-yr | 4241.50 | 2881.45 | 3.10 | 2881.45 | 3.10 | 0 | 0 |
| 1732 | 10-yr | 1175.60 | 2877.79 | 2.61 | 2877.79 | 2.61 | 0 | 0 |
| 1732 | 50-yr | 2366.30 | 2879.26 | 3.06 | 2879.26 | 3.06 | 0 | 0 |
| 1732 | 100-yr | 2923.30 | 2879.82 | 3.14 | 2879.82 | 3.14 | 0 | 0 |
| 1732 | 500-yr | 4241.50 | 2881.22 | 3.07 | 2881.22 | 3.07 | 0 | 0 |
| 1391 | 10-yr | 1175.60 | 2877.21 | 2.52 | 2877.21 | 2.52 | 0 | 0 |
| 1391 | 50-yr | 2366.30 | 2878.95 | 2.22 | 2878.95 | 2.22 | 0 | 0 |
| 1391 | 100-yr | 2923.30 | 2879.58 | 2.20 | 2879.58 | 2.20 | 0 | 0 |
| 1391 | 500-yr | 4241.50 | 2881.08 | 2.14 | 2881.08 | 2.14 | 0 | 0 |
| 869 | 10-yr | 1175.60 | 2876.70 | 2.31 | 2876.70 | 2.31 | 0 | 0 |
| 869 | 50-yr | 2366.30 | 2878.50 | 2.53 | 2878.50 | 2.53 | 0 | 0 |
| 869 | 100-yr | 2923.30 | 2879.17 | 2.59 | 2879.17 | 2.59 | 0 | 0 |
| 869 | 500-yr | 4241.50 | 2880.79 | 2.58 | 2880.79 | 2.58 | 0 | 0 |
| 778 | 10-yr | 1175.60 | 2876.63 | 2.38 | 2876.63 | 2.38 | 0 | 0 |
| 778 | 50-yr | 2366.30 | 2878.44 | 2.67 | 2878.44 | 2.67 | 0 | 0 |
| 778 | 100-yr | 2923.30 | 2879.11 | 2.75 | 2879.11 | 2.75 | 0 | 0 |
| 778 | 500-yr | 4241.50 | 2880.74 | 2.77 | 2880.74 | 2.77 | 0 | 0 |
| 667 | 10-yr | 1175.60 | 2876.59 | 2.16 | 2876.59 | 2.16 | 0 | 0 |
| 667 | 50-yr | 2366.30 | 2878.38 | 2.77 | 2878.38 | 2.77 | 0 | 0 |
| 667 | 100-yr | 2923.30 | 2879.04 | 2.96 | 2879.04 | 2.96 | 0 | 0 |
| 667 | 500-yr | 4241.50 | 2880.67 | 3.15 | 2880.67 | 3.15 | 0 | 0 |
| 501 | 10-yr | 1175.60 | 2876.43 | 2.90 | 2876.43 | 2.90 | 0 | 0 |
| 501 | 50-yr | 2366.30 | 2878.24 | 3.18 | 2878.24 | 3.18 | 0 | 0 |
| 501 | 100-yr | 2923.30 | 2878.92 | 3.28 | 2878.92 | 3.28 | 0 | 0 |
| 501 | 500-yr | 4241.50 | 2880.55 | 3.44 | 2880.55 | 3.44 | 0 | 0 |
| 398 | 10-yr | 1175.60 | 2876.33 | 2.65 | 2876.33 | 2.65 | 0 | 0 |
| 398 | 50-yr | 2366.30 | 2878.14 | 3.05 | 2878.14 | 3.05 | 0 | 0 |
| 398 | 100-yr | 2923.30 | 2878.81 | 3.20 | 2878.81 | 3.20 | 0 | 0 |
| 398 | 500-yr | 4241.50 | 2880.44 | 3.53 | 2880.44 | 3.53 | 0 | 0 |

Appendix B: Estimated Flood Extents

For simplified hydraulic modeling such as HY-8, this section can be omitted.

Include plots/figures (as necessary) to display floodwater extents/comparisons between existing and proposed conditions for all modeled flood events. Indicate water surface elevation differences on exhibits at critical locations along stream channel/reach if necessary.

Flood Extents - 50-yr Peak Discharge (Existing Bridge)

85

Flood Extents – 50-yr Peak Discharge (Proposed Bridge)

85

Appendix C: Structure Comparison Summary

Include SFN Form 18323 (Structure Comparison Summary)

| STRUCTURE COMPARISON SUMMARY |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| North Dakota Department of Transportation, Bridge |  |  |  |  |  |
| SFN 18323 (Rev. 1-2000) |  |  |  |  |  |
| STRUCTURE DATA |  |  |  |  |  |
| ITEM |  | EXISTING | ALT. NUMBER 1 | ALT. NUMBER 1B | ALT. NUMBER 3 |
| TYPE OR DESCRIPTION |  | Cont. Conc. Slab | Quad 12’x12’ RCB | - | - |
| NUMBER OF SPANS, BARRELS, ETC. |  | 2 | 4 | - | - |
| OVERALL LENGTH |  | 65 | 72 | - | - |
| TOTAL WATERWAY AREA (FT2) |  | 514 | 576 | - | - |
| CLEARANCE ELEVATION |  | 1631.0 | 1630.5 | - | - |
| ROADWAY OVERTOPPING ELEVATION |  | 1633.6 | 1634.5 | - | - |
| Hydraulic Comparison |  |  |  |  |  |
| 25-Year Frequency |  | Q = 1835.9 CFS |  |  |  |
| ITEM |  | EXISTING | ALT. NUMBER 1 | ALT. NUMBER 1B | ALT. NUMBER 3 |
| DISCHARGE THROUGH STRUCTURE (CFS) |  | 1835.9 | 1835.9 | - | - |
| DISCHARGE OVER ROADWAY (CFS) |  | 0 | 0 | - | - |
| UPSTREAM STAGE |  | 1627.6 | 1627.1 | - | - |
| DOWNSTREAM STAGE |  | 1626.6 | 1626.6 | - | - |
| AVG. VEL. THROUGH STRUCTURE (FT/SEC) |  | 7.7 | 5.7 | - | - |
| Hydraulic Comparison |  |  |  |  |  |
| 100-Year Frequency |  | Q = 2925.4 CFS |  |  |  |
| ITEM |  | EXISTING | ALT. NUMBER 1 | ALT. NUMBER 1B | ALT. NUMBER 3 |
| DISCHARGE THROUGH STRUCTURE (CFS) |  | 2925.4 | 2925.4 | - | - |
| DISCHARGE OVER ROADWAY (CFS) |  | 0 | 0 | - | - |
| UPSTREAM STAGE |  | 1629.4 | 1628.8 | - | - |
| DOWNSTREAM STAGE |  | 1627.8 | 1627.8 | - | - |
| AVG. VEL. THROUGH STRUCTURE (FT/SEC) |  | 9.7 | 7.9 | - | - |
| Hydraulic Comparison |  |  |  |  |  |
| 200-Year Frequency |  | Q = 4257.7 CFS |  |  |  |
| ITEM |  | EXISTING | ALT. NUMBER 1 | ALT. NUMBER 1B | ALT. NUMBER 3 |
| DISCHARGE THROUGH STRUCTURE (CFS) |  | 4257.7 | 4257.7 | - | - |
| DISCHARGE OVER ROADWAY (CFS) |  | 0 | 0 | - | - |
| UPSTREAM STAGE |  | 1631.2 | 1630.6 | - | - |
| DOWNSTREAM STAGE |  | 1629.0 | 1629.0 | - | - |
| AVG. VEL. THROUGH STRUCTURE (FT/SEC) |  | 11.5 | 9.9 | - | - |
| COMMENTS: | Structure 0030-088.503 |  |  |  |  |
| Alternative 1 includes sinking the box culvert 1’ below the natural channel elevation. |  |  |  |  |  |

Appendix D: Hydraulic Design Data for Bridges/Culverts

Include SFN Form 9634 (Hydraulic Design Data for Bridges) or SFN Form 9636 (Hydraulic Design Data for Culverts) for recommended structure alternatives.

.

HYDRAULIC DESIGN DATA FOR BRIDGES

North Dakota Department of Transportation, Bridge

SFN 9634 (Rev. 1-2000)

| Project Number: IM-8-094(100)337 | Bridge Number: 0094-337.335 L/R |  |
| --- | --- | --- |
| Drainage Area |  | 1,467.8 square miles |
| Design Frequency |  | 50-Year |
| Design Discharge |  | 11,443 cfs |
| Design Stage (upstream) |  | 908.27 ft |
| Stream Gradient (ft/ft) |  | 0.00068 ft/ft |
| Waterway Provided Below Design Stage |  | 2,717. sq ft |
| Waterway Provided Below Clearance Elevation |  | 3291.5 sq ft |
| Average Velocity of Flow in Natural Channel |  | 4.28 ft/sec |
| Depth of Flow |  | 18.36 ft |
| Velocity of Flow Under Bridge |  | 4.21 ft/sec |
| Freeboard Provided |  | 1.94 ft |
| 100-year Frequency Discharge |  | 13,269 cfs |
| 100-year Frequency Stage |  | 909.03 ft |
| Overtopping Stage |  | 914.24 ft |
| Overtopping Discharge |  | 45,093.6 cfs |
| Minimum Water Elevation |  | 889.75 ft |
| COMMENTS: Alternative 3 Bottom Elevation = 889.75 ft, 80' Channel Bottom Width Bridge 0094‐337.335 L ‐ 48' Clear Roadway; Bridge 0094‐337.335 R ‐ 40' Clear Roadway |  |  |

HYDRAULIC DESIGN DATA FOR CULVERTS

North Dakota Department of Transportation, Bridge

SFN 9636 (Rev. 1-2000)

| Project Number: | SS-3-030(036)088 | Bridge Number: |  | 0030-088.503 |
| --- | --- | --- | --- | --- |
| Drainage Area |  |  | 190.661 Sq. Miles |  |
| Stream Gradient (ft/ft) |  |  | 0.001 |  |
| Design Frequency |  |  | 25-Year |  |
| Design Discharge |  |  | 1835.9 cfs |  |
| Design Headwater Stage |  |  | 1627.1 ft |  |
| Design Tailwater Stage |  |  | 1626.6 ft |  |
| Velocity Through Culvert |  |  | 5.7 ft/sec |  |
| 100-year Frequency Discharge |  |  | 2925.4 cfs |  |
| 100-year Frequency Headwater |  |  | 1628.8 ft |  |
| Overtopping Stage |  |  | 1634.5 ft |  |
| Overtopping Discharge |  |  | 6126.5 cfs |  |
| Structure Selection |  |  | Quad 12'x12' RCB |  |
| COMMENTS: Estimated Invert Elevations: Inlet = 1618.5 ft Outlet = 1618.0 ft RCB Skew Angle = 0° (Inverts to be set one foot below the existing channel bottom to provide for aquatic organism passage. The inverts above reflect the one foot sinking of the RCB.) |  |  |  |  |

Appendix E: Watershed Area Map

Include drainage area map with all relevant hydrologic information (drainage area, stream lines, contour lines, basin relief, basin perimeter, stream length/density, Hydrologic Zone, etc).

Appendix F: Structure Inspection Report

Include Structure Inspection Report summary sheet only.

Appendix G: FEMA FIRM Maps

Include FEMA FIRM maps for crossings located within a floodway/floodplain.

Appendix H: Miscellaneous

Include other applicable maps, scour calculation data, survey data, and correspondence as necessary (topographic survey exhibits of critical features such as buildings/drainage structures, scope change documents, email/SOV correspondence, etc).

Alternative 1 – 200-yr Scour Table

Alternative 1 – 200-yr Scour Plot